Probe Z surface:

1. Copy the file M930.m1s to MACH3 the directory  \Mach3\macros\Mach3Mill.

2. Connect the probe:

3.   Input the z offset the Tool change location:

Notice:the z offset(- 63.000 mm) Is higher 5-10MM than the surface (- 68.000 mm) of the probe，

4.  Define the MACH3  Probe  pin.

In the config :ports and pins.define probe pin.

5. in my plugin  define the key function ”Probe Z Surface”.

It will  auto probe z surface when you press the key.

Macro command setting:

In my plugin setting,you input number in the “custom macro number1”,the number is M code.then choose “Custom macro #1”function in the key combox,, you get the M8 function when you press the key.

M8,Food coolant on.

Key define:

use VB Scripting to do M code:

To activate an output5, you should enable output#5 in ports and pins config.then write the following script:

ActivateSignal(OUTPUT5)

Then you save it as m200.m1s   (to signify the output active)  in the directory C:\Mach3\macros\Mach3Mill
To deactivate an output, you write the following script:

DeactivateSignal(OUTPUT5)

Then you save it as m201.m1s   (to signify the output inactive) in the directory C:\Mach3\macros\Mach3Mill

then you could define the m code number 200 and 201 in the “macro number” box.

Custom Oem code setting:

In my plugin setting,you input number in the “Custom OEM CODE #1”,the number is mach3 OEM code.then choose “OEM Button #1”function in the key combox,, you get the code 208 function when you press the key.

208 : Clear Z tool offset (Turn)

Key define: